准备工具：EP7 PRO*1 标配USB线*1

文件包：Icatch V30驱动包*1、EP7L固件刷机包*1

Tools required: EP7 PRO*1, Standard USB cable*1
File package: Icatch V30 driver package*1, EP7L firmware flashing package*1

1.双击Icatch V30驱动包以默认路径安装。

Double-click the Icatch V30 driver package to install it to the default path.

- 打开计算机管理查看通用串行总线控制器内是否已正常安装并显示ICATCH(X) KV驱动

Open Computer Management and check if the ICATCH(X) KV driver is properly installed and displayed under Universal Serial Bus Controllers.

- 双击打开EP7L固件刷机包,双击FRM.exe打开操作界面

Double-click to open the EP7L firmware flashing package, then double-click FRM.exe to open the operation interface.

- 取出EP7PRO和USB数据线，将USB数据线一头接入电脑USB接口，接着先按紧EP7PRO的录像键不放的同时再持续按紧开机键不放，然后再将USB插入EP7PRO的USB接口，这时按着的按键可以放手，点击一下操作界面的ISP(F5)后会自动开始升级

Take out the EP7PRO and USB cable. Connect one end of the USB cable to the computer’s USB port. Then, press and hold the Video button on the EP7PRO while also pressing and holding the Power button. Afterward, plug the USB cable into the USB port on the EP7PRO. Once the buttons are held down, release them, and click ISP (F5) on the operation interface. The upgrade process will automatically begin

- 升级完毕操作界面显示ISP Finished。

After the upgrade is complete, the operation interface will display ISP Finished.